|  | TEST #2 - AR Automation For RSNE Shop |
| --- | --- |

# WORK ORDER

Jobsite

Contacts

Work

| 1. | Cooler Door 3' x 7' |  |  |
| --- | --- | --- | --- |
|  | Work: One (1) 34” x 78” manual swing cooler door and frame; includes self-closing hardware, magnetic gaskets, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (2.50) Magnetic Gasket 8'. (2.00) HINGE D690. (1.00) D90 Handle. (1.00) D276 Door Closer 1-3/8" OS, Satin Chrome. (1.00) EPS Sheet 3.5" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (0.50) Diamond Plate .063 4' x 8'. (0.03) Wiper Gasket 8.5". Labor Scheduled:. n/a. |  |
| 2. | Cooler Door 3' x 7' (99 Restaurants) |  |  |
|  | Work: One (1) 34” x 78” manual swing cooler door and frame; includes premium hardware, neoprene gaskets, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (95.00) Galvanized Steel Coil - Textured White (26ga). (0.03) Wiper Gasket 8.5". (0.20) GASKET, 100' ROLL JAMISON. (3.00) K1245 1-1/4 OS Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) EPS Sheet 3.5" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (1.00) Diamond Plate .063 4' x 8'. (0.25) SSSH 0408 16. (1.00) K1094 Closer. (1.00) Hook for K1094 Closer. Labor Scheduled:. n/a. |  |
| 3. | Cooler Door 4' x 7' |  |  |
|  | Work: One (1) 48” x 84” manual swing cooler door and frame; includes non self-closing heavy-duty hardware, neoprene gaskets, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (3.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (110.00) Galvanized Steel Coil - Textured White (26ga). (0.04) Wiper Gasket 8.5". (0.18) GASKET, 100' ROLL JAMISON. (2.00) K1277 Cam-lift Strap Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) EPS Sheet 3.5" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Diamond Plate .063 4' x 8'. Labor Scheduled:. n/a. |  |
| 4. | Cooler Door 5' x 7' |  |  |
|  | Work: One (1) 60” x 84” manual swing cooler door and frame; includes non self-closing heavy-duty hardware, neoprene gaskets, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (3.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (120.00) Galvanized Steel Coil - Textured White (26ga). (0.05) Wiper Gasket 8.5". (0.20) GASKET, 100' ROLL JAMISON. (3.00) K1277 Cam-lift Strap Hinge. (1.00) K55 Complete. (1.00) EPS Sheet 2" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Diamond Plate .063 4' x 8'. Labor Scheduled:. n/a. |  |
| 5. | Cooler Door High Sill 3' x 7' |  |  |
|  | Work: One (1) 34” x 78” manual swing cooler door and frame; includes self-closing hardware, magnetic gaskets, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (2.50) Magnetic Gasket 8'. (2.00) HINGE D690. (1.00) D90 Handle. (1.00) D276 Door Closer 1-3/8" OS, Satin Chrome. (1.00) EPS Sheet 3.5" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (0.50) Diamond Plate .063 4' x 8'. Labor Scheduled:. n/a. |  |
| 6. | Cooler Slider 4' x 7' |  |  |
|  | Work: One (1) 48” x 84” manual single horizontal sliding cooler door with track system, hardware, neoprene gaskets, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 48. (32.00) Insulated Metal Panel (AWIP)-8'-44-4. (60.00) Galvanized Steel Coil - Textured White (26ga). (0.10) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (4.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 7. | Cooler Slider 5' x 7' |  |  |
|  | Work: One (1) 60” x 84” manual single horizontal sliding cooler door with track system, hardware, neoprene gaskets, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 60. (64.00) Insulated Metal Panel (AWIP)-8'-44-4. (70.00) Galvanized Steel Coil - Textured White (26ga). (0.12) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (5.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 8. | Cooler Slider 6' x 7' |  |  |
|  | Work: One (1) 72” x 84” manual single horizontal sliding cooler door with track system, hardware, neoprene gaskets, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 72. (64.00) Insulated Metal Panel (AWIP)-8'-44-4. (70.00) Galvanized Steel Coil - Textured White (26ga). (0.14) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (5.00) ALUM FLAT. (3.00) Upright Gasket. (70.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 9. | Cooler Slider 6' x 8' |  |  |
|  | Work: One (1) 72” x 96” manual single horizontal sliding cooler door with track system, hardware, neoprene gaskets, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 72. (80.00) Insulated Metal Panel (AWIP)-8'-44-4. (70.00) Galvanized Steel Coil - Textured White (26ga). (0.14) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (6.00) ALUM FLAT. (3.00) Upright Gasket. (70.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 10. | Cooler Slider 8' x 8' |  |  |
|  | Work: One (1) 96” x 96” manual single horizontal sliding cooler door with track system, hardware, neoprene gaskets, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 96. (80.00) Insulated Metal Panel (AWIP)-8'-44-4. (115.00) Galvanized Steel Coil - Textured White (26ga). (0.18) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (8.00) ALUM FLAT. (3.00) Upright Gasket. (80.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 11. | Bottom Sweep Assembly (Cooler) |  |  |
|  | Work: Replacement bottom sweep assembly for cooler doors |  |  |
|  | Materials Required:. (0.03) Wiper Gasket 8.5". (1.20) Galvanized Steel Coil - Textured White (26ga). Labor Scheduled:. n/a. |  |
| 12. | Bottom Sweep Assembly (Freezer) |  |  |
|  | Work: Replacement bottom sweep assembly for freezer doors |  |  |
|  | Materials Required:. (0.03) Wiper Gasket 8.5". (1.20) Galvanized Steel Coil - Textured White (26ga). (1.00) 12' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. Labor Scheduled:. n/a. |  |
| 13. | Cooler Ramp |  |  |
|  | Work: Custom built cooler ramp (40" x 32"). |  |  |
|  | Materials Required:. (0.20) Diamond Plate .063 4' x 8'. (1.00) 2" x 6" x 8’ – Doug Fir. (0.20) TRYMER 200L 4" - 48" X 96". (0.20) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (30.00) Fasteners (Generic). Labor Scheduled:. n/a. |  |
| 14. | EPS Wall/Ceiling Panel (3") - 4 x 10 |  |  |
|  | Work: FRP/EPS RSNE Fabricated Wall/Ceiling Panel - 10' |  |  |
|  | Materials Required:. (2.50) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (1.00) EPS Sheet 2" x 4' x 10'. (2.00) FRP 4 x 10. (0.05) GLUE 5635. Labor Scheduled:. n/a. |  |
| 15. | EPS Wall/Ceiling Panel (3") - 4 x 8 |  |  |
|  | Work: FRP/EPS RSNE Fabricated Wall/Ceiling Panel - 8' |  |  |
|  | Materials Required:. (2.00) 11/32X48X96 ARAUCO AC RADIATA PINE Sanded (3/8"). (1.00) EPS Sheet 2" x 4' x 8'. (2.00) FRP .090 4' x 8' White. (0.02) GLUE 5635. Labor Scheduled:. n/a. |  |
| 16. | EPS Wall/Ceiling Panel (4") - 4 x 10 |  |  |
|  | Work: FRP/EPS RSNE Fabricated Wall/Ceiling Panel - 10' |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (1.00) EPS Sheet 3" x 4' x 10'. (2.00) FRP 4 x 10. (0.05) GLUE 5635. Labor Scheduled:. n/a. |  |
| 17. | EPS Wall/Ceiling Panel (4") - 4 x 8 |  |  |
|  | Work: FRP/EPS RSNE Fabricated Wall/Ceiling Panel - 8' |  |  |
|  | Materials Required:. (2.00) 11/32X48X96 ARAUCO AC RADIATA PINE Sanded (3/8"). (1.00) EPS Sheet 3" x 4' x 8'. (2.00) FRP .090 4' x 8' White. (0.02) GLUE 5635. Labor Scheduled:. n/a. |  |
| 18. | Exterior Cooler Door 3' x 7' |  |  |
|  | Work: One (1) 36” x 84” manual swing cooler door and frame; includes non self-closing heavy-duty lockable hardware, interior/exterior kickplates, neoprene gaskets, security screws, hood, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (0.18) GASKET, 100' ROLL JAMISON. (2.00) K1245 1-1/4 OS Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) EPS Sheet 3.5" x 4' x 8'. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (0.50) Diamond Plate .063 4' x 8'. (1.00) K481 Safety Glow Inside Release, 6" Door. (0.03) Wiper Gasket 8.5". Labor Scheduled:. n/a. |  |
| 19. | Exterior Freezer Door 3' x 7' |  |  |
|  | Work: One (1) 36” x 84” manual swing freezer door and frame; includes non self-closing heavy-duty lockable hardware, neoprene gaskets, security screws, hood, frost-stop heater, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (0.18) GASKET, 100' ROLL JAMISON. (2.00) K1245 1-1/4 OS Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (0.50) Diamond Plate .063 4' x 8'. (1.00) K481 Safety Glow Inside Release, 6" Door. (0.03) Wiper Gasket 8.5". (1.00) 12' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) 32' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 20. | Exterior Freezer Door High Sill 3' x 7' |  |  |
|  | Work: One (1) 36” x 84” manual swing freezer door and frame; includes non self-closing heavy-duty lockable hardware, neoprene gaskets, security screws, hood, frost-stop heater, interior/exterior kickplates, and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (0.20) GASKET, 100' ROLL JAMISON. (2.00) K1245 1-1/4 OS Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (0.50) Diamond Plate .063 4' x 8'. (1.00) K481 Safety Glow Inside Release, 6" Door. (1.00) 40' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 21. | FRP Liner 4 x 10 |  |  |
|  | Work: FRP/EPS/FRP Liner 4' x 10' |  |  |
|  | Materials Required:. (2.00) FRP 4 x 10. (1.00) EPS Sheet 2" x 4' x 10'. (0.05) GLUE 5635. Labor Scheduled:. n/a. |  |
| 22. | FRP Liner 4 x 8 |  |  |
|  | Work: FRP/EPS/FRP Liner 4' x 8' |  |  |
|  | Materials Required:. (2.00) FRP .090 4' x 8' White. (1.00) EPS Sheet 2" x 4' x 8'. (0.05) GLUE 5635. Labor Scheduled:. n/a. |  |
| 23. | Floor Panel (5" urethane) - 8' |  |  |
|  | Work: 8' Floor Panel with 5" urethane foam and .063 diamond plate finish |  |  |
|  | Materials Required:. (1.00) TRYMER 200L 5" - 48" X 96". (1.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (26.00) Galvanized Steel Coil - Textured White (26ga). (0.02) GLUE 5635. (1.00) Diamond Plate .063 4' x 8'. Labor Scheduled:. n/a. |  |
| 24. | Floor Panel - 10' |  |  |
|  | Work: 10' Floor Panel with 3.5" urethane foam and .063 diamond plate finish |  |  |
|  | Materials Required:. (40.00) Galvanized Steel Coil - Textured White (26ga). (1.25) TRYMER 200L 3-1/2" - 48" X 96". (1.25) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (0.03) GLUE 5635. (1.00) Diamond Plate .063 4' x 10'. Labor Scheduled:. n/a. |  |
| 25. | Floor Panel - 8' |  |  |
|  | Work: 8' Floor Panel with 3.5" urethane foam and .063 diamond plate finish |  |  |
|  | Materials Required:. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (1.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (32.00) Galvanized Steel Coil - Textured White (26ga). (0.02) GLUE 5635. (1.00) Diamond Plate .063 4' x 8'. Labor Scheduled:. n/a. |  |
| 26. | Floor Panel - 12' |  |  |
|  | Work: 12' Floor Panel with 3.5" urethane foam and .063 diamond plate finish |  |  |
|  | Materials Required:. (48.00) Galvanized Steel Coil - Textured White (26ga). (1.50) TRYMER 200L 3-1/2" - 48" X 96". (1.50) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (0.05) GLUE 5635. (1.00) Diamond Plate .063 4' x 12'. Labor Scheduled:. n/a. |  |
| 27. | Freezer Door 3' x 7' |  |  |
|  | Work: One (1) 34” x 78” manual swing freezer door and frame; includes self-closing hardware, magnetic gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (95.00) Galvanized Steel Coil - Textured White (26ga). (2.50) Magnetic Gasket 8'. (2.00) HINGE D690. (1.00) D90 Handle. (1.00) D276 Door Closer 1-3/8" OS, Satin Chrome. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (0.50) Diamond Plate .063 4' x 8'. (0.03) Wiper Gasket 8.5". (1.00) 12' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) 34' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 28. | Freezer Door 3' x 7' (99 Restaurants) |  |  |
|  | Work: One (1) 34” x 78” manual swing freezer door and frame; includes premium Kason hardware, neoprene gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (92.00) Galvanized Steel Coil - Textured White (26ga). (3.00) K1248 1-1/4 OS Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) K481 Safety Glow Inside Release, 6" Door. (1.00) Diamond Plate .063 4' x 8'. (0.03) Wiper Gasket 8.5". (1.00) 12' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) 34' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) Kason Slider Exterior Pull Handle. (0.25) SSSH 0408 16. (0.20) GASKET, 100' ROLL JAMISON. Labor Scheduled:. n/a. |  |
| 29. | Freezer Door 4' x 7' |  |  |
|  | Work: One (1) 48” x 84” manual swing freezer door and frame; includes non self-closing heavy-duty hardware, neoprene gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (3.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (110.00) Galvanized Steel Coil - Textured White (26ga). (0.18) Wiper Gasket 8.5". (0.18) GASKET, 100' ROLL JAMISON. (2.00) K1277 Cam-lift Strap Hinge. (1.00) K56 Latch (Body Chrome). (1.00) K56 Strike. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (1.00) 36' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) 15' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Diamond Plate .063 4' x 8'. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 30. | Freezer Door 5' x 7' |  |  |
|  | Work: One (1) 60” x 84” manual swing freezer door and frame; includes non self-closing heavy-duty hardware, neoprene gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (3.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (120.00) Galvanized Steel Coil - Textured White (26ga). (0.05) Wiper Gasket 8.5". (0.20) GASKET, 100' ROLL JAMISON. (3.00) K1277 Cam-lift Strap Hinge. (1.00) K55 Complete. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Diamond Plate .063 4' x 10'. (1.00) 38' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (1.00) 15' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 31. | Freezer Door High Sill 3' x 7' |  |  |
|  | Work: One (1) 34” x 78” manual swing freezer door and frame; includes self-closing hardware, magnetic gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (95.00) Galvanized Steel Coil - Textured White (26ga). (2.50) Magnetic Gasket 8'. (2.00) HINGE D690. (1.00) D90 Handle. (1.00) D276 Door Closer 1-3/8" OS, Satin Chrome. (1.00) TRYMER 200L 4" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (0.50) Diamond Plate .063 4' x 8'. (1.00) 40' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. (0.20) Galvanized Sheet 4' x 8' (20ga). Labor Scheduled:. n/a. |  |
| 32. | Freezer Ramp |  |  |
|  | Work: Custom built heated freezer ramp (40" x 32") |  |  |
|  | Materials Required:. (0.20) Diamond Plate .063 4' x 8'. (1.00) 2" x 6" x 8’ – Doug Fir. (0.20) TRYMER 200L 4" - 48" X 96". (0.20) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (30.00) Fasteners (Generic). (1.00) 15' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. Labor Scheduled:. n/a. |  |
| 33. | Freezer Door Plug 3' x 7' |  |  |
|  | Work: One (1) 34” x 78” manual swing freezer door, no frame; includes self-closing hardware, magnetic gaskets, frost-stop heater, interior/exterior kickplates and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (2.00) 19/32X48X96 ARAUCO AC RADIATA PINE Sanded (5/8"). (60.00) Galvanized Steel Coil - Textured White (26ga). (2.00) Magnetic Gasket 8'. (1.00) TRYMER 200L 3-1/2" - 48" X 96". (0.10) Silicone Sealant (White). (0.20) ADFOAM 1875. (0.05) GLUE 5635. (1.00) Glow Push Panel. (0.03) Wiper Gasket 8.5". (1.00) 12' - HEATER, WIRE ALUM BRAID, 115V, -10 DEG, 2 WRAP. Labor Scheduled:. n/a. |  |
| 34. | Freezer Slider 10' x 10' |  |  |
|  | Work: One (1) 120” x 120” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 120. (144.00) Insulated Metal Panel (AWIP)-8'-44-4. (150.00) Galvanized Steel Coil - Textured White (26ga). (0.24) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (10.00) ALUM FLAT. (3.00) Upright Gasket. (90.00) Fasteners (Generic). (1.00) 42' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. Labor Scheduled:. n/a. |  |
| 35. | Freezer Slider 4' x 7' |  |  |
|  | Work: One (1) 48” x 84” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 48. (32.00) Insulated Metal Panel (AWIP)-8'-44-4. (60.00) Galvanized Steel Coil - Textured White (26ga). (0.10) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (4.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). (1.00) 35.67' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. Labor Scheduled:. n/a. |  |
| 36. | Freezer Slider 5' x 7' |  |  |
|  | Work: One (1) 60” x 84” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 60. (64.00) Insulated Metal Panel (AWIP)-8'-44-4. (70.00) Galvanized Steel Coil - Textured White (26ga). (0.12) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (5.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). (1.00) 35.67' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. (0.25) Galvanized Sheet 4' x 8' (20ga). (1.00) 35.67' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. (0.10) SSSH 0410 26. Labor Scheduled:. n/a. |  |
| 37. | Freezer Slider 6' x 7' |  |  |
|  | Work: One (1) 72” x 84” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 72. (64.00) Insulated Metal Panel (AWIP)-8'-44-4. (80.00) Galvanized Steel Coil - Textured White (26ga). (0.14) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (6.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). (1.00) 35.67' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. Labor Scheduled:. n/a. |  |
| 38. | Freezer Slider 6' x 8' |  |  |
|  | Work: One (1) 72” x 96” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 96. (80.00) Insulated Metal Panel (AWIP)-10'-44-4. (90.00) Galvanized Steel Coil - Textured White (26ga). (0.14) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (6.00) ALUM FLAT. (3.00) Upright Gasket. (60.00) Fasteners (Generic). (1.00) 35.67' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. Labor Scheduled:. n/a. |  |
| 39. | Freezer Slider 8' x 8' |  |  |
|  | Work: One (1) 96” x 96” manual single horizontal sliding freezer door with track system, hardware, neoprene gaskets, frost stop heaters and white embossed galvanized steel finish. |  |  |
|  | Materials Required:. (1.00) SLD 96. (80.00) Insulated Metal Panel (AWIP)-10'-44-4. (120.00) Galvanized Steel Coil - Textured White (26ga). (0.18) Wiper Gasket 3". (1.00) Kason Heavy Duty Floor Roller w/ Strike. (1.00) STRIKER. (1.00) Slider Tongue - Non-padlock. (8.00) ALUM FLAT. (3.00) Upright Gasket. (80.00) Fasteners (Generic). (1.00) 37.5' - HEATER, WIRE FIBERGLASS BRAID, 115V, -10DEG, SINGLE WRAP. Labor Scheduled:. n/a. |  |
| 40. | GALV/FRP Liner 4 x 10 |  |  |
|  | Work: White Embossed GALV/EPS/FRP Liner 4' x 10' |  |  |
|  | Materials Required:. (1.00) FRP 4 x 10. (1.00) EPS Sheet 2" x 4' x 10'. (0.03) GLUE 5635. (40.00) Galvanized Steel Coil - Textured Gray (26ga). Labor Scheduled:. n/a. |  |
| 41. | Heavy Duty Floor Panel - 10' |  |  |
|  | Work: 10' Floor Panel with 5" urethane foam and .125 diamond plate finish |  |  |
|  | Materials Required:. (1.25) TRYMER 200L 5" - 48" X 96". (1.25) 23/32X48X96 ARAUCO AC RADIATA PINE Sanded (3/4"). (40.00) Galvanized Steel Coil - Textured White (26ga). (0.03) GLUE 5635. (1.00) Diamond Plate .125 4' x 10'. Labor Scheduled:. n/a. |  |
| 42. | Heavy Duty Floor Panel - 12' |  |  |
|  | Work: 12' Heavy Duty Floor Panel with 5" urethane foam and .125 diamond plate finish |  |  |
|  | Materials Required:. (1.50) TRYMER 200L 5" - 48" X 96". (1.50) 23/32X48X96 ARAUCO AC RADIATA PINE Sanded (3/4"). (36.00) Galvanized Steel Coil - Textured White (26ga). (0.02) GLUE 5635. (1.50) Diamond Plate .125 4' x 8'. Labor Scheduled:. n/a. |  |
| 43. | Heavy Duty Floor Panel - 8' |  |  |
|  | Work: 8' Floor Panel with 5" urethane foam and .125 diamond plate finish |  |  |
|  | Materials Required:. (1.00) TRYMER 200L 5" - 48" X 96". (1.00) 23/32X48X96 ARAUCO AC RADIATA PINE Sanded (3/4"). (32.00) Galvanized Steel Coil - Textured White (26ga). (0.02) GLUE 5635. (1.00) Diamond Plate .125 4' x 8'. Labor Scheduled:. n/a. |  |
| 44. | Upright Gasket |  |  |
|  | Work: RSNE fabricated upright gasket for sliders |  |  |
|  | Materials Required:. (8.00) POLY FOAM. (8.00) Slider Gasket - 6.5' (Cloth Inserted). (1.00) PVC Corner 8' Black. Labor Scheduled:. n/a. |  |